Town of East Bloomfield

Code Enforcement Office

99 Main Street, PO Box 85

East Bloomfield, New York 14443

Office (585) 657-5455    Fax (585) 657-7276

Subdivision map requirements

for Preliminary Plan

In addition to the requirements for a sketch plan, the following will be required:

_______ 	a. Scale (no smaller than 1" = 50')

_______ 	b. Property boundaries with bearings and distances

_______	c. Existing topography (2' contour intervals maximum). In the case of a relatively level piece of property, a contour interval less than 2' shall be employed to properly study and plan the development.

_______ 	d. Topography datum U.S.C. & G.S.

_______ 	e. Bench marks based on U.S.C. & G.S. datum

_______ 	f. All existing natural features: Watercourses, tree masses, etc

_______ 	g. All existing buildings, culverts, utilities with dimensions, sizes and inverts if possible and other significant man made features.

_______ 	h. All existing property lines, easements or other encumbrances on the property, certified by a licensed" land surveyor per a recent survey and the purpose of which the easements or rights-of-way were established.

_______ 	i. Location and approximate dimensions and sizes of proposed lots (numbered), utilities, streets.

_______  j. Profiles of proposed streets, utilities, etc. with approximate grades.

_______ 	k. Preliminary designs of culverts, pump stations, bridges, sewers, road sections, etc.

_______ 	l. Proposed location, size and width of easements, parks, rights-of-way, public areas or buildings and parcels of land to be dedicated or reserved for public use.

_______ 	m. Zoning setbacks and restriction lines.

_______ 	n. Percolation and deep hole tests and locations with results (if applicable).

_______  o. Special information on details requested by the Planning Board at sketch plan stage.

_______ 	p. Where the Preliminary Plan covers only a part of the developer's entire holdings, a sketch shall be submitted of the prospective street layout for the remainder. Original sketch may be used.

Continue for Final map requirements

Subdivision map requirements

for Final Plan

In addition to the requirements for the Preliminary Plan, the following will be required:

______ a.  Size of the plan and material shall be acceptable for filing in the Ontario County Clerk's Office.

______ b.  Scale (no smaller than 1" = 50')

______ c.  Name and seal of the registered professional engineer, architect or surveyor responsible for the plan.

______ d.  Street lines, lot lines, rights-of-way, easements and areas dedicated or proposed to be dedicated for public use.

______ e.  Sufficient data to determine readily the location, bearing and length of every street, lot and boundary line and to reproduce such lines on the ground.

______ f.  All dimensions shall be shown in feet and in hundredths of a foot.

______ g.  The length of all straight lines, radii, lengths of curves and tangent bearings for each street.

______ h.  All dimensions and angles/bearings of the lines of each lot and each area proposed to be dedicated to public use.

______ i.  The proposed building setback line for each street or the proposed placement of each building.

______ j.  Names of streets within and adjacent to development.

______ k.  Permanent reference monuments shall be shown.

______ l.  Lot numbers and area of each lot.

______ m.  Existing contours (2' maximum depending on topography):

- Proposed finish contours

- Proposed finish garage floor elevations

- Lowest architectural opening elevations in designated flood zone areas

- Spot elevations of swales, etc.

NOTE: On larger projects these should be incorporated on a separate grading plan

______ n.  Location, size, invert elevations, type and class of pipe on all sanitary and storm sewers and manholes.

______ o.  Location, sizes and types of pipe for all watermains, location of all valves, hydrants, blowoffs, etc.

______ p.  Profiles with detailed information of all streets, storm sewers, sanitary sewers and watermain crossings.

______ q.  Design and plan details of all special construction (culverts, bridges, headwalls, etc.)

______ r.  Engineering calculations are required to substantiate the proposed design of utilities, etc.

______ s.  If required, a landscaping plan with planting schedule.

______ t.  Details required to specify special conditions, materials or methods of construction.

______ u.  Indication of approval from jurisdictional agencies.

______ v.  Signature of approval of plans by Town Engineer, Superintendent of Water and Sewer Utilities, Highway Superintendent, Town Board and Planning Board Chairman.

______ w.  Approval of all agencies having jurisdiction over any phase of the development.

______ x.  An affidavit that the applicant is the owner or equitable owner of the land proposed to be developed.

______ y.  A statement duly acknowledged before an officer authorized to take acknowledgement of deeds and signed by the owner or owners of the property, to the effect that the subdivision as shown on the final plan is made with his or their full consent and that it is desired to record the same.

______ z.  An affidavit stating that the applicant will install all improvements shown on the final plan at his own expense.