Town of East Bloomfield

Code Enforcement Office

99 Main Street, PO Box 85

East Bloomfield, New York 14443

Office (585) 657-5455    Fax (585) 657-7276

Subdivision map requirements

for Sketch Plan

The sketch plan map shall be a schematic representation of existing conditions and proposed improvements including:

_____ a. 	Scale (no smaller than 1" = 100')

_____ b. 	Title or name of development

_____ c. 	North point, scale and date

_____ d. 	Name(s) of the record owner(s), developer(s) and their addresses

_____ e. 	Name of Engineer, Surveyor or Architect responsible for preparation of plan

_____ f. 	Zoning District and limitations

_____ g. 	Property boundary dimensions

_____ h. 	Indication of general topography (5' contour intervals) on U.S.C. and G.S. datum

_____ i. 	Indication of all existing physical features (woods, bodies of water, wetlands,

buildings, streams, etc.)

_____ j. 	Indication of all existing utilities, roads, houses, etc.

_____ k. 	Schematic indication of proposed lots (numbered), utilities, roads, etc.

_____ l. 	General location plan (l" = +2, 000')

_____ m. 	Schematic method of water supply, sanitary sewage disposal and surface water drainage

_____ n. 	Names of adjacent owners

_____ o. 	Area of property